## G-1000 MODEL

POLYURETHANE-INJECTED

GARAGE DOOR

ALUMINUM (eq. 23 gauge), door thickness 44.5 mm (1 3/4")

GARAGA INC.								SECTION: 083613

FILE:___________________						Page:_______________

OVERHEAD RETRACTABLE DOORS

WITH METAL PANELS ON HINGES

PART 1: OVERVIEW

1.1	Related work

The author shall give all the necessary references.

1.1.1	Electrical connections, water tightness of openings, and so on.  Remember: all electrical connections and installations must be done by a qualified electrician as recommended by the manufacturer of the electric garage door operators.  (Section 16)

1.1.2	Preparation of the opening of the garage door (Section 5).

1.1.3	These construction specifications and shop drawings are applicable to Garaga Inc. products only.

1.2	Shop drawing

1.2.1	Submit the shop drawings as per the instructions outlined in Section 01300.

1.2.2	The shop drawings shall include all the following information: the type of materials, the type of opening mechanism, the required tolerances, the electrical connections, the structural fastenings executed by the general contractor, and the suitable matching with neighboring materials.

1.3	Maintenance records

1.3.1	Provide the necessary instructions to ensure proper operation and maintenance of all the hardware components for the doors as well as the electric garage door operators, and include these instructions with the manual on use and maintenance described in Section 01300.

1.4	Qualifications

1.4.1	The manufacturer of materials shall be registered in accordance with ISO-9001 (2000 version) standards.  The installation of this section shall be fabricated by a manufacturer with a minimum of 7 years experience in the actual production of the specified products.

1.4.2	The installation shall be executed by a company approved by the garage door manufacturer as an installer, using skilled installers experienced on this work.

PART 2: GARAGE DOORS

2.1	Calculation criteria

2.1.1	The doors and the hardware system shall be designed to meet or exceed the industry standard for windload (Door & Access Systems Manufacturer Association).

(Note: for doors wider than 5.5 m (18’), or high wind situations, consult our engineering service).

2.1.2	The doors shall have a thermal resistance factor of RSI 2.8 (k = 0.357 W/m2K).

2.1.3	The doors, the tracks, and the springs shall be designed to withstand at least ________operation cycles per year and _________total cycles over their useful life.

Note: the number of spring cycles can vary from 10 000 to 100 000 cycles. However, you must realize that you cannot get high cycles for every size of door. If you want the best operating system, with the highest cycles possible, please specify “springs shall be designed for the maximum operation cycles”.

2.2	MATERIALS

2.2.1	Enameled aluminum

Aluminum sheet metal of 0.60 mm (0.02”) thick finished in the manufacturer’s standard colours in accordance with, or exceeding, the following standards:

1.1	Protective film thickness:			A.S.T.M.	D.1400

1.2	Hardness of the finished surface:		C.G.S.B.	93.G.P.1

1.3	Saline air resistance:				A.S.T.M.	B.117

1.4	Moisture resistance:				A.S.T.M.	D.2247

1.5	Sheen:						A.S.T.M.	D.525

1.6	Immersion in water:				A.S.T.M.	D.870

1.7	Resistance to fading:				A.S.T.M.	D.659

The aluminum sheet metal surface shall be of the woodgrain type with two (2) or four (4) decorative horizontal grooves.

2.2.2	Insulation

High-pressure, CFC-Free, polyurethane foam has been injected between the walls of each panel.  Its density is 40.4 Kg/m3 (2.5 lb/in.3) with a thermal resistance factor of RSI 1.6 per 25 mm (1”) of thickness.  The total insulation factor is R-16, RSI 2.8 (k = 0.357 W/m2K) (A.S.T.M. standard C-518-91). This insulation is in accordance with standards ONGC 51-GP-21M and 51.26-M86.

2.2.3	Reinforcements

14 gauge steel reinforcement plates are inserted within the door panels to provide proper fastening for such hardware accessories as the handle, the hinges, and the plate for the garage door operator.

2.2.4	Panel ends

A block of dry pine (grade 4) is inserted at both ends of each insulated garage door section for the fastening of the lateral hinges.

2.2.5	Regular windows (if applies)

Clear double thermopane windows shall have a total thickness of 22.2 mm (7/8”).  The 3 mm (1/8”) panes are sealed in aluminum extrusions.  The windows are inserted in an expanded PVC frame and factory installed by the manufacturer.

or	Panoramic windows ( if applies )

Clear double thermopane windows shall have a total thickness of 16 mm (5/8”).  The 3 mm (1/8”) panes are sealed in aluminum extrusions.  The windows are inserted in (clear anodized) (white) aluminum tubular extrusions and measure 1.6 mm (0.06”) thick.  The windows are fixed with rigid PVC moldings and brackets.  They are factory installed by the manufacturer.

2.3	DOORS

2.3.1	The garage doors shall be of the model G-1000 as made by Garaga Inc.  The panels are shaped with 0.60 mm (0.02”) aluminum sheet and electronically injected with high-pressure polyurethane for a total minimum thickness of 44.5 mm (1.8").

2.3.2	The doors shall have the following sizes and features:

DOORS	            SIZES		  # OF SECTIONS	                  # OF PANES

LOCATION	         (W. by H.)		     WITH PANES		    PER SECTION

__________	___________	________________		_________________

__________	___________	________________		_________________

__________	___________	________________		_________________

2.4	WEATHER TIGHTNESS

2.4.1	Provide and install continuous weatherstripping at the bottom of the lower panel.  The weatherstripping shall be made of a U-shape aluminum extrusion as well as a semi-circular E.P.D.M. rubber tubing.

2.4.2	Inset weatherstripping of flexible and rigid PVC shall be found at the intersection of each panel.  This type of weatherstripping will ensure an efficient thermal barrier as well as double weather tightness in accordance with the following standards: when submitted to a pressure of 0.075 kPa, which is equivalent to winds of 40 km/h, the air infiltration rating as measured using standard A.S.T.M. E-283 shall be of 0.033 liter/second per meter of joint between the door sections.

2.4.3	At the head of the door, provide the top of the panel with continuous weatherstripping made up of a strip of flexible PVC of 65 mm (0.03”) in width.

2.4.4	Provide and install, on the exterior side of the door jambs and lintel, weatherstripping made up of an aluminum extrusion as well as a double-edged strip of arctic vinyl.  This weatherstripping shall be adjustable and come with rigid PVC screw cover.

2.5	OPTIONS (to choose from)

2.5.1	End caps that are made of 16 gauge galvanized steel of at least 2.5 mm (1”) thick shall be installed at the end of each section where the hinges are to be fastened.

2.5.2	Exhaust ports of 100 mm (4”) in diameter are factory installed on each door in order to place 	flexible hoses for evacuating exhaust gases.

PART 3: HARDWARE WITH TORSION SPRINGS

3.1	PRODUCTS

3.1.1	The tracks

The tracks are made of 12 gauge (2.6 mm (0.1”)) galvanized steel of 76 mm (3”).  The horizontal track is reinforced with a steel angle of 50 x 50 mm (2" X 2”).

3.1.2	Hardware

The hinges are made of 13 gauge galvanized steel.  Industrial rollers of 75 mm (3”) with 10 ball- bearings are used.

3.1.3	Struts for large doors (if applies)

Doors measuring in excess of 3759 mm (12'-4") in width will come with 22 gauge galvanized steel horizontal struts, and 13 gauge double hinges at each end.

3.1.4	Type of movement

The movement of the hardware will allow for the most space possible available underneath the door when it is in the open position.

DOORS	    	      TYPE OF MOVEMENT	 	  AVAILABLE SPACE	                LOCATION	       					 	   (FLOOR/CEILING)

____________		     _____________________	           ______________________

____________		     _____________________	           ______________________

____________		     _____________________	           ______________________

3.1.5	Torsion-type springs

The torsion spring lifting system will consist of all the parts and accessories needed for its installation.  All doors weighting more than 454 kg (1000 lb), hardware parts included, must be approved by a professional in installation in regards to the choice of pieces of hardware (drums, galvanized cables, springs, anchor plate, solid shaft of 25 mm (1”)).

3.2	OPTIONS (to choose from)

3.2.1	75 mm (3”) rubber rollers, long (for apartment buildings only)

Garage door rollers for 75 mm (3”) tracks will have a U.H.M.V. head. They also include a sealed 6202-2RS precision ball bearing and are mounted on an SAE 1020 galvanized steel rod.

3.2.2	Track guards

The vertical tracks are protected by track guards (in an L shape 1524 mm x 6.4 mm (5’ x 0.3”) or in a Z shape 1524 mm x 5.0 mm (5’ x 0.2”)) in order to avoid accidental breaking.  Often, track guards are painted with a light colour after the installation.

3.2.3	Chain hoist

Doors of over 3 m (10’) high with manual operation come equipped with a chain hoist mounted on the wall.

Note: If electric operator, consult specs in the next section.

3.2.4	Pusher springs (to choose from)

For doors exceeding 15 m2 (161 ft2), the standard movement hardware will come with pusher springs at the end of the horizontal tracks in order to prevent the cables from falling off the drums.

3.2.5	"C" bumper springs

For dock doors (ex: 2438 mm x 2438 mm (8’ x 8’)), the vertical lift movement hardware will come with “C” bumper springs at the end of the vertical tracks in order to prevent the cables from falling off the drums.

3.2.6	Flanged bearing

The end bearing plates will come equipped with flanged bearings for doors weighting more than 300 kg (660 lb). Center plate (football bearings) will be included with doors of this weight.

3.2.7	Tension bridge reinforcements

Doors exceeding 5563 mm (18'-3") will come equipped with tension bridge reinforcement.  They will be mounted at each end of the top panel and held in the center by a support whose height is adjusted according to the width of the door.  These reinforcements must be installed according to the Garaga Inc. exact specifications.

Note: all doors which weight exceeds 454 kg (1000 lb), including hardware parts attached to the door, must be designed according to manufacturer’s specifications. Consult the Garaga technical service department.

CHOOSE:	- Trolley-type operators for commercial use

- Jackshaft-type operators

- Trolley-type operators for apartment buildings

PART 4:	TROLLEY-TYPE OPERATORS FOR COMMERCIAL USE

4.1	PRODUCTS

4.1.1	These are trolley-type electric operators that come equipped with a quick-release device which instantly disconnects the door from the operator to enable manual operation in the event of a power failure.  The system consists of a carriage that slides between dual galvanized steel angle tracks.

Note: that 100 mm (4”) of free space is required between the highest point of the door travel and the ceiling.

4.1.2	The electric motors, control mechanisms, relays, and electrical devices of the operator shall be approved according to CSA and UL standards.

4.1.3	The electrical power supply is of ____________ volts, _______ phase(s) and 60 Hz.

DOORS		         MODEL 		HORSEPOWER	 * TYPE OF

LOCATION								 MOVEMENT

______________	________________	  ___________		____________

______________	________________	  ___________		____________

______________	________________	  ___________		____________

* Standard lift only

4.1.4	The operators come equipped with an instantly reversible motor, which has built-in thermal protection mechanism. The electrical control circuit shall be of 24 volts.

4.1.5	The door shall travel at approximately 200 to 280 mm/sec (7.9 to 11 in/sec.).

4.1.6	The control panel for the door operator shall be of the push-button type for Up/Down/Stop control and surface mounted on the inside wall.

4.1.7	A safety mechanism with photocells shall be added to each operator in order to stop and reverse the movement of the door in the event an object cuts off the light beam.

AND / OR

An electro-pneumatic safety device with an instant reverse feature will be added to each operator.  It consists of a rubber hose concealed within the weatherstripping located at the bottom of the door, combined with the electric switches.  This device makes the door stop and go back up as soon as it comes into contact with an object.

4.2	OPTIONS (to choose from)

4.2.1	The door operators identified by the numbers _____ come with a remote control system.

4.2.2	A control station with a key switch for Up/Down/Stop shall be surface mounted on the outside of the building.

4.2.3	All side latches will be equipped with an electrical interlock switch to prevent use of the electric operator when the door is locked.

PART 4:	JACKSHAFT-TYPE OPERATORS

4.1	PRODUCTS

4.1.1	The electric operators shall be of the Jackshaft type and shall come equipped with a built-in chain hoist assembly and with a quick release mechanism to allow for manual operation of the door in the event of a power failure.

4.1.2	The electric motors, control mechanisms, relays, and electrical devices of the operator shall be approved according to CSA and UL standards.

4.1.3	The electrical power supply is of ____________ volts,  _______ phase(s) and 60 Hz.

DOORS		         MODEL 		HORSEPOWER	 * TYPE OF

LOCATION								 MOVEMENT

______________	________________	  ___________		____________

______________	________________	  ___________		____________

______________	________________	  ___________		____________

4.1.4	The operators shall feature a positive lock mechanical brake, an instantly reversible motor, and a thermal protection device against power surges. The electric control circuit is of 24 volts.

4.1.5	The door shall travel at a speed of 200 to 230 mm/sec (7.9 to 11 in/sec).

4.1.6	The control panel for the door operator shall be of the push-button type for Up/Down/Stop control and surface mounted on the inside wall.

4.1.7	A safety mechanism with photocells shall be added to each operator in order to stop and reverse the movement of the door in the event an object cuts off the light beam.

AND / OR

An electro-pneumatic safety device with an instant reverse feature will be added to each operator.  It consists of a rubber hose concealed within the weatherstripping located at the bottom of the door, combined with the electric switches.  This device makes the door stop and go back up as soon as it comes into contact with an object.  It also comes with a 1.5 second delay on reverse feature which helps prevent too much bouncing on the lifting cables when the door is going back up.

4.2	OPTIONS (to choose from)

4.2.1	The door operators identified by the numbers _____ come with a remote control system.

4.2.2	A control station with a key switch for Up/Down/Stop shall be surface mounted on the outside of the building.

4.2.3	All side latches will be equipped with an electrical interlock switch to prevent use of the electric operator when the door is locked.

PART 4:	TROLLEY-TYPE OPERATORS FOR APARTMENT BUILDINGS

4.1	PRODUCTS

4.1.1	These are trolley-type electric operators that come equipped with a quick-release device, which instantly disconnects the door from the operator to enable manual operation in the event of a power failure.  The system consists of a carriage that slides between dual galvanized steel angle tracks.

Note: that 100 mm (4”) of free space is required between the highest point of the door travel and the ceiling.

4.1.2	The electric motors, control mechanisms, relays, and electrical devices of the operator shall be approved according to CSA and UL standards.

4.1.3	The electrical power supply is of ____________ volts,  _______ phase(s) and 60 Hz.

DOORS		         MODEL 		HORSEPOWER	 * TYPE OF

LOCATION								 MOVEMENT

______________	________________	  ___________		____________

______________	________________	  ___________		____________

______________	________________	  ___________		____________

* Standard lift only

4.1.4	The operators come equipped with an instantly reversible motor, which has built-in thermal protection mechanism.  The electrical control circuit shall be of 24 volts.

4.1.5	The door shall travel at a speed of 150 to 180 mm/sec (6 to 7.1 in/sec).

4.1.6	The control panel for the operator comes with a single button for opening, closing, and reversing the movement of the door.  This control panel is surface mounted on the inside wall.

4.1.7	A safety mechanism with photocells shall be added to each operator in order to stop and reverse the movement of the door in the event an object cuts off the light beam.

    - A mechanism consisting of green and red warning lights as well as automatic closing feature control the operation of the door.  After the door arrives in its fully open position, it will remain open for an adjustable preset time.  Then it will close automatically.  The warning lights will ensure control of vehicular traffic.

  - OPTIONS (to choose from)

4.2.1	The door operators identified by the numbers _____ come with a remote control system.  The total number of transmitters required is of ______ units.

4.2.2	A control station with a key switch for opening the door or a wireless keyless entry system shall be mounted on a steel post outside the apartment building.  It shall be placed in such a way as to be accessible to the users from their vehicle.

4.2.3	A control station for use of a card scanner shall be mounted on a steel post outside the apartment building.  It shall be placed in such a way as to be accessible to the users from their vehicles.  ______ cards shall be provided with the operator.

4.2.4	A pull cord switch shall be mounted near the opening of the door.  It will enable manual opening and closing of the door.  It will also reset the timer for automatic closing (Note: if the motor comes with this option).

4.2.5	A magnetic loop detector shall be installed in the floor.  When a vehicle or a metal object approaches the inductive loop, the detector sends a signal to the electric operator to signal the presence of a vehicle.  The signal of the magnetic detector can be coupled with the card scanner control station (4.2.3).  In this way, the presence of a vehicle will be necessary for the card scanner to open the garage door.

PART 5: INSTALLATION

5.1	Before starting, make sure that the frames and the fixtures prepared by the general contractor are square.

5.2	Install the doors and the related hardware.

5.3	Apply some touch-up paint to areas where the finish might have been damaged during the mounting.

5.4	Install the electric motors, control devices, push-button control stations, relays, and other electrical equipment needed for operating the door.

5.5	All electrical connections must be done by a certified electrician.

5.6	Adjust all movable parts in such a way as to get good weather tightness against all conditions.

5.7	Adjust the weatherstripping with the exterior jambs in such a way as to get good weather tightness against all conditions.

5.8	Make sure all of the mechanisms that have been installed work properly.

  - Clean doors as recommended by the manufacturer, and get rid of all leftover materials and debris found near the openings and the hardware.